.pl 70

.op

.he        TMS9900/99105 Cross-assembler  Rev. 1.0                    Page #

TMS9900/99105 Cross-assembler

Rev. 1.0

Derived from the original William C. Colley, III.

M6800 Cross Assembler

by

Alexander Cameron

Written and modified during June 1984.

The Manual Such As It Is.

.pa

.pn 1

.op

...COLUMNWIDTH 65

...MARGINWIDTH 6

...T1

1.1       Format of Cross-assembler Commands

...T2

1.1.1     Command Strings

To   run  the 9900 cross-assembler,   type  the

following command line:

A>a99 filename options

filename:

The   name   of  the  source  input   file   is

filename.a99,  the list can go to filename.l99,

and the hex file can go to filename.h99.

options:   See next section.

...T2

1.1.2     Options

The   source  file  comes  from  the  currently

logged  disk drive  unless  it is redirected by

putting  the  string "sd" in the options field.

s  specifies the source file and d is a  device

code from the following list:

a, b, c, d   Disk drives.

-            The currently logged in disk drive.

Lines   of input containing errors will  always

be  output to the console device.   If the full

listing  is desired,  it must be called for  by

putting  the string "ld" in the options  field.

l specifies list file and d is a device code as

per the source file,  or one of the  following:

x              Console device.

y              List device.

The   hex file will not be output unless called

for  as per the list file.

Options   must  be run together into  a  single

string with no embedded blanks.

...T2

1.1.3     Examples

A>a99 barf       source -- a:barf.a99

list   -- none.

hex    -- none.

A>a99 barf sblxha source -- b:barf.a99

list   -- con:

hex    -- a:barf.h99

A>a99 barf ly    source -- a:barf.a99

list   -- lst:

hex    -- none.

A>b:a99 barf sbh-  source -- b:barf.a99

list   -- none.

hex    -- a:barf.h99

...T1

1.2       Format of TMS9900 Cross-assembler Source Files

Lines of source input are terminated with CR/LF

pairs.  Internally, the character before the LF

is discarded, so if the CR is missing, the last

character  of the line will be eaten  up.   The

internal line buffer will hold a 120  character

line.    This  may be changed by  altering  the

"#define  LINLEN  120" statement in  xasm99.gbl

and  recompiling the cross-assembler.    Excess

characters in the line are thrown out.

Lower   case   letters are converted  to  upper

case in  as few places as possible.  They are:

1)  In opcodes,

2)  In checking for keywords such as NOT, and

3)  In command strings.

This  means that "not",   "NOT",   "Not",  etc.

are   all  possible  spellings of  the  logical

inversion   operator.  This  also   means  that

"foo"   and   "FOO"   are   different  symbols.

Watch  this if you start encountering U  errors

of symbols that you "know" you defined.

...T2

1.2.1     Statements

Source files input to the 9900  Cross-assembler

consist of statements of the form:

[label] [opcode] [arguments] [;comments.]

Labels   are  recognized by their beginning  in

column  1. If  it  doesn't  begin there,  it is

assumed  to   be   an  opcode.     Labels   are

assigned  the  current  program counter   value

unless  the line's opcode is  "EQU"  or "SET".

Opcodes    may   be  either  9900   instruction

mnemonics  or pseudo-ops.

The   arguments following the opcode will  vary

with  the opcode.   In the case of opcodes such

as "NOP", they may be absent entirely.

Lines  terminate with either a CR/LF pair or  a

semicolon  that  is  not embedded in a   quoted

string.   Anything after a semicolon is ignored

by  the  assembler,  but  will  appear  in  the

listing.   Note  that  a semicolon in column  1

will make the entire line a comment.

...T2

1.2.2     Symbols

Symbols  may  be of any length,   but only  the

first   8 characters  are  significant.    This

may  be  changed   by  changing   the  "#define

SYMLEN   8"   statement    in    a68.gbl    and

recompiling   the   assembler.      Note   that

this  increases   the storage required  by  the

symbol  table   as each  entry  in  the  symbol

table  is  SYMLEN+2  bytes   in  length.    The

following    characters   are   legal   in    a

symbol:

A-Z  a-z  !  &  .  :  ?  [  \  ]  ^  _  `

{  |  }  ~  0-9

Note   that  symbols may not begin with 0-9  as

this would make  them impossible to distinguish

from Intel  format numbers.

A   special   symbol $ is always equal  to  the

address  of the first byte of code generated by

a given line.

...T2

1.2.3     Numeric Constants

Numbers  begin  with  0-9,%.  %  leading   base

designator  represent  binary.  If  the  number

begins with 0-9,  the  assembler  looks for one

of the trailing  base designators B,  O,  Q, D,

or  H.   If the number does not end with one of

these,  it is assumed to be decimal.  B is  the

base designator for binary,   O and Q  are  for

octal,   D   is  for decimal,   and H  is   for

hexidecimal.  Trailing base designators and the

hex digits A-F can be in  either upper or lower

case.    Note  that  hex numbers  that    start

with   A-F   and   are   specified   with   the

designator   H  must have a leading zero  added

to  keep them from being mistaken for  symbols.

For example:

0ff80h 		   evaluate to ff80 hex.

128 and 128d             evaluate to 80 hex.

35o, and 35q             evaluate to 1d hex.

%0111000 and 0111000b    evaluate to 39 hex.

A  character  constant is a string composed  of

zero,  one, or two ASCII characters,  delimited

by  either single or double  quotes ('  or  ").

Note  that  single quotes only  balance  single

quotes,  and double quotes only  balance double

quotes.   Thus  a character constant of 22  hex

can be specified by '"'.  For example:

"ab"           evaluates to 6162 hex.

""             evaluates to 0000 hex.

"'"            evaluates to 0027 hex.

'A'            evaluates to 0041 hex.

Note   that  in  the  two-character   character

constant,   the left-hand character will appear

in  the  upper-order byte and   the  right-hand

character will appear in the lower-order byte.

...T2

1.2.4     Strings

Strings   are   formed  in  the  same  way   as

character constants  except that they may be of

any length up  to and including 255 characters.

The  first  character  will be  placed  in  the

lowest-order   memory   byte.     A   line-feed

character  may  not  be  embedded in  a  string

as   it terminates  the line and will  yield  a

quote   imbalance error.    Note  that  strings

are  only  valid  in  the arguement field of an

"TEXT" pseudo-op.

...T1

1.3       Expression Evaluation

The   following   operators  are  allowed    in

expressions.  The operators are listed in order

of precedence.

Unary Plus, Unary Minus

*, /, MOD, SHL, SHR

+, -

>, >=, <, <=, <>, =

(These can also be written as

GT, GE, LT, LE, NE, EQ.)

NOT       (1's complement)

AND       (Bitwise logical AND)

OR, XOR   (Bitwise logical OR and Exclusive

OR)

HIGH, LOW

Parentheses  are  used to change the  order  of

precedence.     During     evaluation   of   an

expression,   as  soon as a  new  operator   is

encountered  that has precedence less  than  or

equal   to   the  last  operator   encountered,

all  operations  up  to the  new  operator  are

performed.      That    is,      subexpressions

involving   operators  of    higher  precedence

are computed first.

All operators except +,   -,  *,  /, =, <>, >=,

<=, >, < must  be separated from their operands

by at least  one space or tab.

The  "HIGH mumble" is equivalent to "mumble SHR

8"   and "LOW mumble" is equivalent to  "mumble

AND 0ffH".

The relational operators (=,  >, etc.) evaluate

to 0ffffh if the relationship is true,  0 if it

is false.

Expressions     are   terminated   by   commas,

semicolons,  or CR/LF pairs.

...T1

1.4       990 Family Instructions

The     instructions    of    the    9900/99105

microprocessor  fall  into several  catagories.

Each requires a certain  set  of arguments  and

disallows  other arguments.   Arguments may  be

specified  in  any order and are  separated  by

commas    Blanks    may    be    used    freely

throughout   the argument field (except  within

symbols  or operators such as  NOT) to  enhance

the readability of the source code.

The basic syntax is as follows:

[label]   LI  R1,expression   ;The expression

;is evaluated

;from left to

;right.

The  type  of  addressing each  instruction  is

allowed  along with the number of operands  the

assembler  expects the instruction to have,  is

carried in the ATTRIB byte which is returned by

the  GETOPC  function.    The  ATTRIB  byte  is

is  made  up  of two 4 bit  nibbles  the  least

significat  describing operand 1 and  the  most

significant  describing  operand  2.  (See  the

heading comments of GETOPC which is part of the

file A99TBLS.CSM).    The ATTRIB byte will also

tell  the  assembler which one of the  7  basic

instruction  formats a particulare  instruction

falls into.   Thes formats/groups are discussed

in the following sections.

...T2

1.4.1     Addressing Modes

The  9900/99105 instruction set allows 5  basic

addressing  modes.  These are:

Mode                          Example

----                          -------

Workspace register            MOV R1,R3

Workspace register indirect   A *R1,R2

Symbolic                      CLR @expr

Indexed                       A @expr(R1),R2

Workspace register indirect   DEC *R7+

(auto increment)

These modes will in general be written in  the

form:

R, *R, *R+, @X, @X(R)

where  R  refers  to  a Register and  X  to  an

address.

...T2

1.4.2     Instruction Formats

The  assembler will properly evaluate  7  basic

instrucion formats; these are:

Format 0  -  No operands at all (N)

Format 1  -  R,*R,*R+,@X,@X(R)

Format 2  -  Displacement (Disp)

Format 3  -  Destination Field only (D)

Format 4  -  Signed Displacement (SD)

Format 5  -  Workspace Register (W or C)

Format 6  -  Immediate Operands (IOP)

Format 7  -  Bit Manipulation Instrucions

(TMS99105 only)

Depending  on the instruction these formats may

lie  within  either the source  or  destination

fields.

...T2

1.4.3     Group 1 Instructions

This group contains the following opcodes:

NOP

RSET IDLE CKOF CKON LREX

RTWP

These  opcodes permit no arguments,  and  hence

are of format 0.

...T2

1.4.4     Group 2 Instructions

This group of instructions are of Format type 1

and are Dual Operand Instructions with Multiple

Addressing  Modes  for Source  and  Destination

Operand.

This group contains the following opcodes:

A  AB C CB S SB SOC SOCB

SZC  SZCB  MOV MOVB

Instruction format:  [opcode|format1|format1]

Example:             A *R1+,R2

...T2

1.4.5     Group 3 Instructions

This  group  of instructions are  Dual  Operand

Instructions with Multiple Addressing Modes for

the  Source  Operand  and  Workspace   Register

Addressifng for the Destination.

This group contains the following opcodes:

COC CZC XOR MPY DIV XOP LDCR STCR

Instruction format:  [opcode|format3|format1]

Example:              XOP @LABEL,3

...T2

1.4.6     Group 4 Instructions

This   group  of  instructions  allow  Multiple

Addressing  modes for the source  Operand,  and

thus a single operand instructions.

This  following  instructions  belong  to  this

group:

B BL BLWP CLR SETO INV NEG ABS SWPB

INC INCT DEC DECT X

BIND(a TMS99105 instruction)

Instruction format:  [opcode|format1]

Example:             INC @LABEL(R3)

...T2

1.4.7     Group 5 Instructions

This  group of instructions are CRU  Single-Bit

Instructions.

The  following  instructions  belong  to   this

group:

SBO SBZ TB

Instruction format:  [opcode|format4]

Example:             SBO 25

1.4.8     Group 6 Instructions

This   group   covers  the   Conditional   Jump

instrucions.     Note    that    the    maximum

displacement range is +127 to -128 words.

The   following  instructions  belong  to  this

group:

JEQ JGT JH JHE JL JLE JLT JMP JNC JNE

JNO JOC JOP

Instruction format:  [opcode|format2]

Example:             JEQ LABEL

...T2

1.4.9     Group 7 Instructions

This  group  have an Immediate  Source  Operand

with  an Workspace Register as the  Destination

Operand.

The   following  instructions  belong  to  this

group:

AI ANDI CI LI ORI

BLSK (a TMS99105 instruction)

Instruction format:   [opcode|format5]

[    format6   ]

Example:              AI R3,128

...T2

1.4.10    Group 8 Instructions

This  group  covers the Internal Register  Load

Immediate  Instructions and have  an  Immediate

Source Operand only.

The  following  instructions  belong  to   this

group:

LWPI LIMI

Instruction format:   [ opcode  ]

[ format6 ]

Example:              LIMI 3

...T2

1.4.11    Group 9 Instructions

This  group covers the Internal Register  Store

Instructions  and  have  Destination  Workspace

Register Addressing only.

The   following  instructions  belong  to  this

group:

STST STWP

LST  LWP ( TMS99105 instructions )

Instruction format:   [opcode|format5]

Example:              STST R3

...T2

1.4.12    Group 10 (TMS99105 only)

This group covers the Bit-Manipulation instruc-

tions peculiar to the TMS99105.  This group has

a  different instruction format than the  basic

9900  set  insofar as the format fields lie  in

the 2nd word.   The instructions in this  group

are:

TMB TCMB TSMC

Instruction format:   [     opcode     ]

[ format3|format1]

Example:              TMB @BITMAP(R3),8

...T1

1.5       Pseudo-operations

...T2

1.5.1     END

When   this  statement  is   encountered,   the

assembler  decides   that   the   end  of   the

source   file  has  been reached.     If   this

statement   is  missing  or  in  a  conditional

assembly block that is not being assembled, the

assembler  will  encounter end-of-file  on  the

source  file.    In this case,   the  assembler

will  attach an END statement  and will flag an

error.   The syntax  is  as follows:

[label]   END

If  any IF statement is not closed with an ENDI

at this point, an error will be flagged.

This statement permits no arguments.

...T2

1.5.2     EQU

This  statement is used to assign  a  permenant

value  to  a symbol.    This value may  not  be

subsequently changed by a SET,  another EQU, or

by  writing the symbol in column 1 as a  label.

The syntax is as follows:

label     EQU  expression

A   phasing  (P)  error  will  result  if   any

forward  references   are  encountered  in  the

evaluation  of  the expression.

...T2

1.5.3     BYTE

This  statement  is  used  to  place  bytes  in

memory.     Each   byte    is  defined  by   an

expression.    Multiple bytes can be defined by

expressions  separated  by  commas.     If   an

expression  evaluates to a quantity outside the

range -128  to 255,  an error is flagged.   The

syntax  is  as follows:

[label]   BYTE  [expr1][,epxr2].....

...T2

1.5.4     TEXT

This  statement  is used to  place  strings  in

memory.   Its  arguments  may only be  strings.

Multiple strings may be placed  in  memory   if

the  strings  are  separated  by  commas.   The

syntax is as follows:

[label]   TEXT  [string][,string].....

...T2

1.5.5     WORD

This  statement  is  used  to  place  words  in

memory.     Each   word    is  defined  by   an

expression.    Multiple words may be defined by

expressions separated by  commas.    Words  are

placed  in memory with their high-order byte in

the low-order memory location and the low-order

byte  in the high-order memory  location.   The

syntax is as follows:

[label]   WORD  [expr1][,expr2].....

...T2

1.5.6     BSS (Block Starting with Symbol)

This  statement is used to reserve a  block  of

memory   for  working   storage,    etc.     It

requires one argument that gives the number  of

bytes  of storage to  reserve.    The syntax is

as follows:

[label]   BSS  expression

...T2

1.5.7     EVEN

This  statement  is used to force the assembler

to  align code on an even word boundary if  the

present  programme  counter  is  odd.   If  the

programme counter is odd a zero byte is  output

if   even  the  statement  is   ignored.    The

assembler will also flag an error is an attempt

is made to output code to an odd address.   The

syntax is as follows:

[label]   EVEN

...T2

1.5.8     DXOP

This statement allows the programmer to  define

their   own   macro  instructions  around   the

Extended  Operation  Instruction  (XOP).    The

assemble  will  substitute the XOP code  for  a

given instruction in place of the name assigned

to the XOP.  The syntax is as follows:

DXOP PUSH,3    ;assign the name PUSH to

;XOP number 3

For  example if PUSH is a previously coded  XOP

which  pushes  registers  onto the  stack  then

instead  of coding this XOP instruction as  XOP

R3,3  you can code PUSH R3 - this will  produce

the  same code as XOP R3,3 but will   generally

make  the  programme far more easy  to  follow.

The 990 CPU's allow up to 15 XOP's.

...T2

1.5.9     Conditional Assembly

Blocks   of  code  can  be  assembled  or   not

assembled    based  on   the  value   of   some

expression.   The basic syntax is as follows:

IF   expression

(lines of code)

ENDI

If  the  expression evaluates to 0,   the  code

will not be assembled.    The source lines will

be  transfered  to the  listing,   but  no  hex

output will be generated.    It  is recommended

that   a  value  of $ffff be used for  a   true

value  since it turns into a false value  under

bitwise inversion.

In    addition,     the   ELSE   directive   is

supported  as follows:

IF   expression

(lines of code)

ELSE

(more lines of code)

ENDI

This is equivalent to:

IF   expression

(lines of code)

ENDI

IF   NOT expression

(more lines of code)

ENDI

Note   that   labels  are  not   permitted   on

conditional  assembly directives.    Also  note

that   an  END   statement  can   fail   to  be

recognized if it is in  a  conditional assembly

block with a false expression.

A  phasing  (P) error will be flagged if  there

are  any forward references in the expression.

...T2

1.5.10    AORG

This   statement   is  used  to load  a   value

into  the assembly program counter.   The value

is  obtained  from the  expression.    If   the

expression  contains   forward  references,   a

phasing  (P) error will result.  If a label  is

present,  it  will be EQUed to the new  program

counter value.  The syntax is:

[label]   AORG  expression

...T2

1.5.11    SET

This  statement is used to assign  a  temporary

value  to  a symbol.    The  symbol may not  be

redefined by an EQU or by  writing it in column

1  as a label,   but it  may  be  redefined  by

another  SET  statement.    The value  for  the

symbol is obtained from the expression.   If it

contains  forward  references,   a phasing  (P)

error will  result.

The syntax is:

label     SET  expression

...T1

1.6       Error Messages

Error   messages   are flagged with  a   single

letter   in column  1  of the offending line in

the   listing.    The  meaning of  each  letter

follows.

A    Presently not implemented.

B    Distance  on  a  branch   instruction

is    too   great.     Use   a   jump

instruction  with a branch around it,

or rearrange your code to shorten the

distance.

D    Digit  too  large for  the  base  was

encountered.   In particular,   watch

for 8 or 9 in an  octal number and a-

f in a decimal number.

E    Expression ill-formed.   Look for non

arithmetic   characters  in   indexed

addressing modes.

I    If  stack imbalance.    Look for ELSE

or   ENDI  without   an IF or  an  IF

still open at the end of the file.

L    Invalid  label.    Label may  contain

invalid characters  or  be equal to a

reserved  word like NOT.    Label may

be present on  a conditional assembly

directive.

M    Label   multiply   defined.     Label

defined   more  than  once  and   all

definitions are not SET statements.

O    Invalid     opcode.       Look    for

misspellings,  missing semicolons, or

opcodes in column 1.

P    Phasing error.   Look for expressions

in   ORG,    EQU,    SET,    or    IF

directives    that   contain  forward

references.  If none exist, something

wierd is going on as a label that was

defined   in  pass   1   mysteriously

vanished in pass 2.

R    Register value too large.   Registers

values  between  0  and 15  only  are

allowed.  This error will most likely

occur  if a register value  is  being

evaluated  from  an expression  which

overflows.

S    Syntax  error.    Check  your  syntax

against  my samples.

T    Too     many   arguments   on    this

line.     WORD  directives  may  only

define  127  words,   while TEXT  and

BYTE directives may only define   255

bytes.      On    other   statements,

you    put  superfluous arguments  on

the line.

U    Undefined     symbol      encountered

during expression evaluation.

V    Value     out    of   bounds.      In

particular,   FCB expressions  and 8-

bit  immediate values  must  evaluate

to -128 to 255,   while index  values

must evaluate to 0 to 255.

*    This   statement  generated  by   the

assembler.   Right   now,   this only

happens   if   you   drop   an    END

statement      or      remove      it

through conditional assembly.

"    Quote   imbalance  error.    Bear  in

mind  that  ' will  not  terminate  a

string  started   with  "  and  vice-

versa.

(    Parenthesis  imbalance error.   Count

them!

...T1

1.7       Assembler Abort Conditions

Under  certain circumstances,   this  assembler

will  just give up and quit in the middle of an

assembly.    If you don't  get the error  count

diagnostic on  the  console, read  your  screen

for sure!   The  following  messages occur:

1)   Can't open source.

The    source  file  does  not  exist

on  the specified disk drive.

2)   Can't open list.

Can't open hex.

No   directory  entries left  on  the

disk  drive in question.

3)   Illegal command line.

Bone up on command lines.

4)   No file info supplied.

Bone up on command lines.

5)   If stack overflow.

IF   directives  may only  be  nested

16   deep.    Rebuild   your   source

code  to  reduce  your nesting depth,

or  change  the "#define IFDEPTH  16"

statement in xasm99.gbl and recompile

the assembler.

6)   Disk read error.

Source   file   has  a  bad  CRC   or

some  other difficulty.

7)   Disk write error.

Out of disk or directory space on the

list or hex file.

8)   Error closing file.

Problem    closing   list   or    hex

file.    You shouldn't get this one.

9)   Symbol Table Overflow.

Your  source program defines too many

symbols.   The  current limit is 500.

If  you have more memory than  I  do,

you  can change the "#define  SYMBOLS

500" line in a99.gbl  and   recompile

the    assembler.    Otherwise,   you

must work on your source code.

...T1

1.8       Compiling the Assembler

To   compile the assembler as it  stands,   you

will  need the following items:

1)   40K of RAM.

2)   The BDS C Compiler Version 1.5

(good box, Leor!!).

3)   Digital   Research's   ASM   or   MAC

assembler.   (Note:   Only  needed if

you want to play  with the  functions

in  a99tbls.mac as they  exist in   a

form  digestible  by  CLINK  in   the

file xasm99.crl.)

To  get  a99tbls.crl  up  from  scratch,    you

do   the  following.    I   assume   all  files

including ASM or MAC live on drive A.

A>casm a99tbls        ;source a99tbls.csm

This should yield a file a99tbls.asm on drive A.

A>mac  a99tbls $pz -s  ;may use asm fn.aaz

A>cload  a99tbls       ;produce CRL file

You  should  now have a99tbls.crl on  drive  A.

Now  you  are ready to compile the rest of  the

beast.

A>cc a99

A>cc a99asmlnc

A>cc a99evalc

A>cc a99getc

A>cc a99putc

A>cc a99symbc

Now you can link it all together.

A>clink a99 -s

*a99asmln

*a99eval

*a99get

*a99put

*a99symb

*a99tbls

And, as if by magic, you've got a99.com!

Note  that  the linkage can all be done on  one

line,   but my paper isn't wide enough here  to

do  it  that way.  Note  that if you have  more

than 40K of  RAM,   you  may want  to  increase

your symbol table size.    Symbols are 10 bytes

each, so plan accordingly.

...T1

1.9       Final Comments

Happy   assembling!    If you have questions or

note   any bugs,    I'd   appreciate  a   shout

at    (617)258-1204,  (617)266-3179  or on  the

Boston CBBS at  (617)963-8310.   The  questions

will  point up weaknesses in the   manual,  and

the bugs need to be stamped upon unmercifully.

#############################################