The following was used to connect a vga monitor to the Geneve

XRGB-2 UPSCAN CONVERTER

WIRE    6 STRAND

8-PIN DIN PLUG    RADIO SHACK #274-026

SCART PLUG      (PERITEL PLUG)    (EURO ADAPTER)

NEC MULTISYNC 3D MONITOR

MODEL #JC-1404HMA

Monitor is a D series

The XRGB-2 is available from NCSX.COM

Under Accessories for Dreamcast or Playstation.

Signal	21-pin Connector

Red	15

red shield	13

Green	19

green shield	17

blue	20

blue shield	18

Sync	9

Sync shield	7

Left Audio	5

Left shield	3

Right Audio	1

Right shield	3

+5v	16

Ground	21

XRGB-2 PINOUTS