|||||||||||||||||||||||||

|||||||||||||||||||||||||

|||||||||||||||||||||||||

|||||||||||||||||||||||||

ASM7 ASSEMBLER

USER'S MANUAL

|||||||||||||||||||||||||

|||||||||||||||||||||||||

Copyright 1988            |||||||||||||||||||||||||

Texas Instruments          |||||||||||||||||||||||||

All rights reserved.        |||||||||||||||||||||||||

|||||||||||||||||||||||||

The information and/or drawings    |||||||||||||||||||||||||

set forth in this document  and    |||||||||||||||||||||||||

all rights in and to inventions    |||||||||||||||||||||||||

disclosed  herein  and  patents    |||||||||||||||||||||||||

which might be granted  thereon    |||||||||||||||||||||||||

disclosing   or  employing  the    |||||||||||||||||||||||||

materials, methods, techniques,    |||||||||||||||||||||||||

or  apparatus described  herein    |||||||||||||||||||||||||

are the  exclusive  property of    |||||||||||||||||||||||||

Texas Instruments.                 |||||||||||||||||||||||||

|||||||||||||||||||||||||

No disclosure of information or    |||||||||||||||||||||||||

drawings shall  be made to  any    |||||||||||||||||||||||||

other  person  or  organization    |||||||||||||||||||||||||

without  the prior  consent  of    |||||||||||||||||||||||||

Texas Instruments.                 |||||||||||||||||||||||||

|||||||||||||||||||||||||

Texas Instruments Incorporated     |||||||||||||||||||||||||

Consumer Products Division         |||||||||||||||||||||||||

P.O. Box 650311  MS 3920           |||||||||||||||||||||||||

Dallas, TX 75265                   |||||||||||||||||||||||||

|||||||||||||||||||||||||

TEXAS INSTRUMENTS

INCORPORATED

|||||||||||||||||||||||||

|||||||||||||||||||||||||

Document: ASM7 V4.0                |||||||||||||||||||||||||

Date: 19 SEP 88                    |||||||||||||||||||||||||

|||||||||||||||||||||||||

TABLE of CONTENTS

TABLE of CONTENTS

Paragraph                    Title                           Page

SECTION 1 Introduction                               1-1

1.1     Revision . . . . . . . . . . . . . . . . . . . . . . 1-1

1.2     Overview . . . . . . . . . . . . . . . . . . . . . . 1-1

SECTION 2 Running ASM7                               2-1

2.1     Command Line Options . . . . . . . . . . . . . . . . 2-1

2.1.1     B  . . . . . . . . . . . . . . . . . . . . . . . . 2-1

2.1.2     D  . . . . . . . . . . . . . . . . . . . . . . . . 2-1

2.1.3     F  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.4     L  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.5     N  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.6     O  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.7     R  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.8     T  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.9     X  . . . . . . . . . . . . . . . . . . . . . . . . 2-2

2.1.10    W  . . . . . . . . . . . . . . . . . . . . . . . . 2-3

2.2     Source File Format . . . . . . . . . . . . . . . . . 2-3

2.2.1     Constant Bases . . . . . . . . . . . . . . . . . . 2-3

2.2.2     Expressions  . . . . . . . . . . . . . . . . . . . 2-3

2.3     Object File Format . . . . . . . . . . . . . . . . . 2-3

2.4     Listing File Format  . . . . . . . . . . . . . . . . 2-4

SECTION 3 ASM7 Directives                            3-1

3.1     Location Counter Directives  . . . . . . . . . . . . 3-1

3.1.1     AORG . . . . . . . . . . . . . . . . . . . . . . . 3-1

3.1.1.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-1

3.1.1.2     Description  . . . . . . . . . . . . . . . . . . 3-1

3.1.2     BES  . . . . . . . . . . . . . . . . . . . . . . . 3-1

3.1.2.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-1

3.1.2.2     Description  . . . . . . . . . . . . . . . . . . 3-2

3.1.3     BSS  . . . . . . . . . . . . . . . . . . . . . . . 3-2

3.1.3.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-2

3.1.3.2     Description  . . . . . . . . . . . . . . . . . . 3-2

3.1.4     DORG . . . . . . . . . . . . . . . . . . . . . . . 3-2

3.1.4.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-2

3.1.4.2     Description  . . . . . . . . . . . . . . . . . . 3-3

i

TABLE of CONTENTS

3.1.5     EVEN . . . . . . . . . . . . . . . . . . . . . . . 3-3

3.1.5.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-3

3.1.5.2     Description  . . . . . . . . . . . . . . . . . . 3-3

3.1.6     RORG . . . . . . . . . . . . . . . . . . . . . . . 3-3

3.1.6.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-3

3.1.6.2     Description  . . . . . . . . . . . . . . . . . . 3-3

3.2     Assembler Output Directives  . . . . . . . . . . . . 3-4

3.2.1     IDT  . . . . . . . . . . . . . . . . . . . . . . . 3-4

3.2.1.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-4

3.2.1.2     Description  . . . . . . . . . . . . . . . . . . 3-4

3.2.2     LIST . . . . . . . . . . . . . . . . . . . . . . . 3-4

3.2.2.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-4

3.2.2.2     Description  . . . . . . . . . . . . . . . . . . 3-4

3.2.3     OPTION . . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.2     Description  . . . . . . . . . . . . . . . . . . 3-5

3.2.3.3     B  . . . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.4     D  . . . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.5     F  . . . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.6     R  . . . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.7     T  . . . . . . . . . . . . . . . . . . . . . . . 3-5

3.2.3.8     X  . . . . . . . . . . . . . . . . . . . . . . . 3-6

3.2.3.9     W  . . . . . . . . . . . . . . . . . . . . . . . 3-6

3.2.4     PAGE . . . . . . . . . . . . . . . . . . . . . . . 3-6

3.2.4.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-6

3.2.4.2     Description  . . . . . . . . . . . . . . . . . . 3-6

3.2.5     TITL . . . . . . . . . . . . . . . . . . . . . . . 3-6

3.2.5.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-6

3.2.5.2     Description  . . . . . . . . . . . . . . . . . . 3-7

3.2.6     UNL  . . . . . . . . . . . . . . . . . . . . . . . 3-7

3.2.6.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-7

3.2.6.2     Description  . . . . . . . . . . . . . . . . . . 3-7

3.3     Constant Initialization Directives . . . . . . . . . 3-7

3.3.1     BYTE . . . . . . . . . . . . . . . . . . . . . . . 3-7

3.3.1.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-7

3.3.1.2     Description  . . . . . . . . . . . . . . . . . . 3-8

3.3.2     DATA . . . . . . . . . . . . . . . . . . . . . . . 3-8

3.3.2.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-8

3.3.2.2     Description  . . . . . . . . . . . . . . . . . . 3-8

3.3.3     EQU  . . . . . . . . . . . . . . . . . . . . . . . 3-8

3.3.3.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-8

3.3.3.2     Description  . . . . . . . . . . . . . . . . . . 3-8

3.3.4     RBYTE  . . . . . . . . . . . . . . . . . . . . . . 3-9

3.3.4.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-9

3.3.4.2     Description  . . . . . . . . . . . . . . . . . . 3-9

3.3.5     RDATA  . . . . . . . . . . . . . . . . . . . . . . 3-9

3.3.5.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-9

3.3.5.2     Description  . . . . . . . . . . . . . . . . . . 3-9

3.3.6     RTEXT  . . . . . . . . . . . . . . . . . . . . . . 3-10

3.3.6.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-10

3.3.6.2     Description  . . . . . . . . . . . . . . . . . . 3-10

ii

TABLE of CONTENTS

3.3.7     TEXT . . . . . . . . . . . . . . . . . . . . . . . 3-10

3.3.7.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-10

3.3.7.2     Description  . . . . . . . . . . . . . . . . . . 3-10

3.4     Program Linkage Directives . . . . . . . . . . . . . 3-11

3.4.1     DEF  . . . . . . . . . . . . . . . . . . . . . . . 3-11

3.4.1.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-11

3.4.1.2     Description  . . . . . . . . . . . . . . . . . . 3-11

3.4.2     REF  . . . . . . . . . . . . . . . . . . . . . . . 3-11

3.4.2.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-11

3.4.2.2     Description  . . . . . . . . . . . . . . . . . . 3-11

3.5     Miscellaneous Directives . . . . . . . . . . . . . . 3-12

3.5.1     COPY . . . . . . . . . . . . . . . . . . . . . . . 3-12

3.5.1.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-12

3.5.1.2     Description  . . . . . . . . . . . . . . . . . . 3-12

3.5.2     END  . . . . . . . . . . . . . . . . . . . . . . . 3-12

3.5.2.1     Fields . . . . . . . . . . . . . . . . . . . . . 3-12

3.5.2.2     Description  . . . . . . . . . . . . . . . . . . 3-12

SECTION 4 Further Information                        4-1

iii

TI-74 ASM7                                           Introduction

SECTION 1

Introduction

1.1 Revision

This   is   revision   4.0   of    this    package,    dated

September 21, 1988.

1.2 Overview

ASM7  is  a  PC-based  assembler  for  the TMS7000 family of

microcomputers.  ASM7 is full featured, although it does not have

a macro capability.  This document will cover the  features  that

the assembler provides, and how to use this assembler.

Texas Instruments            1-1

TI-74 ASM7                                           Running ASM7

SECTION 2

Running ASM7

ASM7  is  executed  by  entering  the  'ASM7'  command to the DOS

prompt.  ASM7 can assemble individual files, or  multiple  files,

and  can  accept  wildcard  filename specifications (ie. *.ASM to

assemble all assembly files in one directory).

Syntax for the command line to ASM7 is:

C:\>ASM7 -options file1,file2,...

The command line options shown above may or may not be used,  and

the  filenames  specified  may  be full DOS pathnames (with drive

specification) or a  filename  in  the  current  directory.   The

filenames may also contain wildcard specifications.

2.1 Command Line Options

ASM7   accepts  command  line  options  of  the  form  "ASM7

-option_letter...".  The various command line options that may be

specified are documented in the following paragraphs. Options may

be specified in either upper or lower case to the assembler.

2.1.1 B

The -B option causes BYTE and RBYTE directives to only  list  the

first  4  bytes  specified.   If not specified, then all BYTE and

RBYTE directives cause all of their constants to be listed in the

listing file.

2.1.2 D

The -D option causes DATA and RDATA directive to  only  list  the

first 2 words of data specified, although all data specified will

be included in the object module file.

Texas Instruments            2-1

TI-74 ASM7                                           Running ASM7

2.1.3 F

The  -F option is identical to specifying -BDT to disable listing

of more than 4 bytes of information to the listing file  for  any

BYTE, RBYTE, DATA, RDATA, TEXT, and RTEXT directive.

2.1.4 L

The  -L  option disables listing file generation completely. When

specified, no listing file will  be  created  from  the  assembly

process.

2.1.5 N

The  -N  option causes line numbers to be included in the listing

file if one is generated.  The default is to not add line numbers

to the listing file.

2.1.6 O

The -O option disables creation of the output object  module  for

the  assembled  module.  When specified, no object module will be

created.  The default is to write the object module.

2.1.7 R

The -R option causes a reduced cross  reference  map  of  symbols

encountered  in  the assembly process to be written.  The default

is to not write a cross reference map.

2.1.8 T

The -T option causes TEXT and RTEXT directives to only generate 4

bytes of information  in  the  listing  file,  although  all  the

specified data will be included in the object module.

2.1.9 X

The  -X option causes generation of a full cross reference map to

be included at the end of the listing file.  The  default  is  to

not write a cross reference map.

Texas Instruments            2-2

TI-74 ASM7                                           Running ASM7

2.1.10 W

The -W option causes the listing file to not include any assembly

warnings  that  occurred.   Errors will still be listed after the

line that they occurred in.

2.2 Source File Format

The input source file should use a .ASM extension on the PC.

All labels start  in  column  one  of  the  file,  with  assembly

directives  and  mnemonics  preceded by at least one blank space.

Labels cannot have comments after  them  unless  they  are  first

followed by an EQU directive.

2.2.1 Constant Bases

Constants  in  the  ASM7 assembler are by default, in the decimal

base.  To  force  a  constant  into  another  base,  one  of  the

following indicators should precede the constant:

>    : hexadecimal constant

#    : hexadecimal constant

?    : binary constant

'    : starts and ends string (text) constant

2.2.2 Expressions

Expressions  in  ASM7  consist of numeric constants, symbols, and

operators.  The valid list of operators and their  functions  are

listed in the following table:

+    : addition

-    : subtraction

*    : multiplication

/    : integer division

%    : integer modulus

&    : logical AND function

|    : logical OR function

^    : logical XOR function

2.3 Object File Format

The  object  file  created  will  contain  TI  tagged object

Texas Instruments            2-3

TI-74 ASM7                                           Running ASM7

format.   Refer  to  the  Object  Format  Document  for   further

information on this format.

2.4 Listing File Format

The  listing  file  will contain a single line of output for

each input line, with the relocatable (or  absolute)  address  of

that instruction followed by the opcode data for that instruction

at  the  beginning  of  the  line.   If the line number option is

specified, then the line will be preceded by the line number  for

that line.

Texas Instruments            2-4

TI-74 ASM7                                        ASM7 Directives

SECTION 3

ASM7 Directives

ASM7  version  4.0  supports  24  directives  that  allow various

functions to be  performed.   These  directives  are  covered  in

detail in this section.

3.1 Location Counter Directives

3.1.1 AORG

Syntax: [<label>] AORG <word-expression> [<comment>]

3.1.1.1 Fields

The  label  is  optional  and if used, is assigned the same value

that AORG places in the location counter.

The  word  expression  is  evaluated  and  assigned  to  the

location counter

3.1.1.2 Description

AORG  loads  the  location  counter  with  the   value   of   the

word-expression.   This  directive causes the object module to be

an absolute (not relocatable) module.

3.1.2 BES

Syntax: [<label>] BES <word-expression> [<comment>]

3.1.2.1 Fields

The label field is optional and if used, is assigned the value of

Texas Instruments            3-1

TI-74 ASM7                                        ASM7 Directives

the location counter at the end of the reserved block of data.

The operand field is evaluated, and the value added  to  the

location counter.

3.1.2.2 Description

BES  evaluates  the operand expression and adds this value to the

location counter.  This directive will  therefore  allocate  byte

buffers  of  various  sizes,  with the last byte address location

assigned to the optional label attached to the directive.

3.1.3 BSS

Syntax: [<label>] BSS <word_expression> [<comment>]

3.1.3.1 Fields

The label field is optional and will be assigned the value of the

location counter at the lowest address of the  block  created  by

the directive.

The operand is evaluated and is used by the directive as the

value to add to the location counter.

3.1.3.2 Description

The  BSS  directive  adds  the  expression  value to the location

counter.  This directive will therefore allocate byte buffers  of

various  sizes,  with the first byte address location assigned to

the optional label attached to the directive.

3.1.4 DORG

Syntax: [<label>] DORG <expression> [<comment>]

3.1.4.1 Fields

The label is optional, and if used, is assigned  the  same  value

that will be placed in the location counter.

The  operand field contains an expression that can be either

relocatable or absolute.

Texas Instruments            3-2

TI-74 ASM7                                        ASM7 Directives

3.1.4.2 Description

DORG loads the location counter with the starting  address  of  a

dummy block or section.

3.1.5 EVEN

Syntax: [<label>] EVEN [<comment>]

3.1.5.1 Fields

The  label  field  for the EVEN directive is optional and will be

assigned the value of the location counter after the directive is

processed if used.

3.1.5.2 Description

This directive will force the location counter to the  next  word

address  boundary.   If the location counter is already at such a

location, no alteration is performed.

3.1.6 RORG

Syntax: [<label>] RORG <expression> [<comment>]

3.1.6.1 Fields

The label is optional, but if used is  assigned  the  same  value

that will be assigned to the location counter.

The  operand  used  must be a relocatable expression and can

contain only previously defined symbols.

3.1.6.2 Description

RORG places the  relocatable  value  of  the  expression  in  the

location counter.

Texas Instruments            3-3

TI-74 ASM7                                        ASM7 Directives

3.2 Assembler Output Directives

3.2.1 IDT

Syntax: [<label>] IDT 'string' [<comment>]

3.2.1.1 Fields

The  label is optional, but if used is assigned the current value

of the location counter.

The operand contains a module name in  the  'string'.   This

string may be upto 8 characters long.

3.2.1.2 Description

IDT  assigns a name to the object module produced by the assembly

process.  This name can be seen in the link map file produced  by

TILINK to identify the module easily.

3.2.2 LIST

Syntax: [<label>] LIST [<comment>]

3.2.2.1 Fields

The label is optional, but if used takes the value of the current

location counter.

3.2.2.2 Description

LIST  restores  printing of the listing to the listing file if it

was cancelled by a previous UNL directive.

Texas Instruments            3-4

TI-74 ASM7                                        ASM7 Directives

3.2.3 OPTION

Syntax:    OPTION <option-list>

3.2.3.1 Fields

Labels are not used with the OPTION  directive,  and  no  comment

should follow an OPTION directive.

3.2.3.2 Description

The  OPTION  directive  may  select  several  options relating to

assembly listing output.  The operand is a list of  keywords  (or

letters)  separated  by  commas.   Each option selects one of the

following features (and may be selected by command line option as

well; see section above.)

3.2.3.3 B

The B option limits the listing of BYTE and RBYTE  directives  to

one line.

3.2.3.4 D

The  D  option limits the listing of DATA and RDATA directives to

one line.

3.2.3.5 F

The F option limits  the  listing  of  all  constant  initializer

directives  (BYTE,  RBYTE,  DATA,  RDATA, TEXT, and RTEXT) to one

line.  It is the same as using OPTION B,D,T.

3.2.3.6 R

The R option causes a reduced cross-reference table of symbols to

be produced in the listing file.

3.2.3.7 T

The T option limits the listing of TEXT and RTEXT  directives  to

Texas Instruments            3-5

TI-74 ASM7                                        ASM7 Directives

one line.

3.2.3.8 X

The X option causes a full cross-reference table of symbols to be

produced in the listing file.

3.2.3.9 W

The  W  option  causes the exclusion of warning messages from the

listing file.

3.2.4 PAGE

Syntax: [<label>] PAGE [<comment>]

3.2.4.1 Fields

The label is optional, but will be  assigned  the  value  of  the

location counter if present.

3.2.4.2 Description

The  PAGE  directive  causes  a page eject in the source listing,

with listing immediately continued on the next page.

3.2.5 TITL

Syntax: [<label>] TITL 'string'

3.2.5.1 Fields

The label field is optional, but will be assigned  the  value  of

the current location counter if present.

The 'string' field contains the assembly title to be used on

line 2 of each page of the listing file.  Up to 60 characters may

be specified in the title string.

Texas Instruments            3-6

TI-74 ASM7                                        ASM7 Directives

3.2.5.2 Description

The  TITL  directive  supplies  a page title for each page of the

assembly listing, and can occur more than once in a  module  (for

different  page  titles).   The title will be printed on the next

page after the TITL directive is encountered.

3.2.6 UNL

Syntax: [<label>] UNL  [<comment>]

3.2.6.1 Fields

The label field is optional and will be assigned the value of the

location counter if present.

3.2.6.2 Description

The UNL directive stops  program  listing  output  until  a  LIST

directive  is  encountered.  This is useful when assembling files

with many included COPY files that do not require listing.

3.3 Constant Initialization Directives

3.3.1 BYTE

Syntax: [<label>] BYTE <exp>[,<exp>] [<comment>]

3.3.1.1 Fields

The label field is  optional,  but  takes  the  current  location

counter value if present.

The operand field contains one or more expressions separated

by commas.  These expressions cannot contain external references.

The assembler evaluates these expressions and places the value in

a  byte.   If  truncation  is  required,  the  assembler prints a

truncation warning and puts the LSB of the value in the byte.

Texas Instruments            3-7

TI-74 ASM7                                        ASM7 Directives

3.3.1.2 Description

The BYTE directive places one or  more  values  in  one  or  more

successive bytes of memory.

3.3.2 DATA

Syntax: [<label>] DATA <exp>[,<exp>] [<comment>]

3.3.2.1 Fields

The  label  field  is  optional,  but  takes the current location

counter value if present.

The operand field contains one or more expressions separated

by commas.  These expressions can  contain  external  references.

The assembler evaluates these expressions and places the value in

a  word  as  a  16-bit number, most significant byte first (lower

address).

3.3.2.2 Description

The DATA directive places one or more words of  data  in  one  or

more successive 2-byte words of memory.

3.3.3 EQU

Syntax: <label> EQU <expression> [<comment>]

3.3.3.1 Fields

The  label  field  is required and contains a symbol that will be

assigned the operand's value.

The operand is an expression whose value will be assigned to

the label.  The operand may not  contain  a  REF'd  symbol  or  a

forward reference.

3.3.3.2 Description

The EQU directive assigns a value to a symbol.

Texas Instruments            3-8

TI-74 ASM7                                        ASM7 Directives

3.3.4 RBYTE

Syntax: [<label>] RBYTE <exp>[,<exp>] [<comment>]

3.3.4.1 Fields

The  label  is optional and will be assigned the current location

counter value if present.

The operand field contains one or more expressions that will

be evaluated, and their values treated as 8-bit numbers, assigned

to one or more successive byte locations.

3.3.4.2 Description

The RBYTE directive is similar to the BYTE directive except  that

the 8-bit value is stored in reverse digit fashion.  For example,

if  the  value  >A6  is  given  as  the  expression  to  the BYTE

directive, the location will contain >A6.  This same value  given

the RBYTE directive will contain >6A (nibble reversal).

3.3.5 RDATA

Syntax: [<label>] RDATA <exp>[,<exp>] [<comment>]

3.3.5.1 Fields

The  label  is  optional  but  will  be assigned the value of the

current location counter if present.

The operand field contains one or more expressions that will

be evaluated to a 16-bit number, and  assigned  in  byte-reversed

order to successive byte locations.

3.3.5.2 Description

The RDATA directive is similar to the DATA directive, except that

instead  of  storing  the data words in the form MSB at the lower

address, the RDATA directive stores the data word in the form LSB

at the lower address.

Texas Instruments            3-9

TI-74 ASM7                                        ASM7 Directives

3.3.6 RTEXT

Syntax: [<label>] RTEXT [-]'string' [<comment>]

3.3.6.1 Fields

The label is optional and will be assigned the  current  location

counter value if present.

The  operand  contains  the  string  data  to  be  stored in

successive bytes of memory in reverse order (ie. first  character

of  string stored at highest byte).  If the string is preceded by

a unary minus, the highest addressed character will have its most

significant bit set (same as or'ing it with 128).

3.3.6.2 Description

The RTEXT directive stores a string in reverse order  in  memory,

with  the  last  character  of  the  operand  field at the lowest

address in memory.

3.3.7 TEXT

Syntax: [<label>] TEXT [-]'string' [<comment>]

3.3.7.1 Fields

The label is optional and will be assigned the  current  location

counter value if present.

The  operand  contains  the  string  data  to  be  stored in

successive bytes of memory in the order encountered in the string

(ie. first character stored  at  the  lowest  address).   If  the

string  is  preceded  by  a  unary  minus,  the highest addressed

character will have its most significant bit set (same as  or'ing

it with 128).

3.3.7.2 Description

The  TEXT  directive  stores  a string in normal order in memory,

with the last character of  the  operand  field  at  the  highest

address in memory.

Texas Instruments            3-10

TI-74 ASM7                                        ASM7 Directives

3.4 Program Linkage Directives

3.4.1 DEF

Syntax: [<label>] DEF <symbol>[,<symbol>] [<comment>]

3.4.1.1 Fields

The  label  is optional and if present takes the current value of

the location counter.

The operand field contains one or more symbols, separated by

commas, to be defined in the program  being  assembled.   If  the

symbols are not encountered in the module, the assembler produces

and undefined symbol error for each symbol.

3.4.1.2 Description

DEF  makes  one or more symbols available to other programs.  All

symbols used in the DEF statement must be  defined  in  the  same

module.   The  symbols  encountered  in the DEF statement will be

included in the object module output file.

3.4.2 REF

Syntax: [<label>] REF <symbol>[,<symbol>] [<comment>]

3.4.2.1 Fields

The label field is optional and will be given the  value  of  the

current location counter.

The operand field contains one or more symbols, separated by

commas,  defined  in  other  modules,  that are to be used in the

current module.

3.4.2.2 Description

REF provides access to one  or  more  symbols  defined  in  other

programs.   An  error occurs at link time if the REF'd symbol has

Texas Instruments            3-11

TI-74 ASM7                                        ASM7 Directives

not been DEF'd in another module.

3.5 Miscellaneous Directives

3.5.1 COPY

Syntax: [<label>] COPY <filename> [<comment>]

3.5.1.1 Fields

The label is optional and will be given the current value of  the

location counter if present.

The  filename in the operand field must correspond to a file

that exists on the system being  used  to  assemble  the  module.

Nested copying of files is not allowed with this release of ASM7.

3.5.1.2 Description

COPY changes the source input for the assembler to the file named

in  the  operand  field.   When END-OF-FILE is encountered on the

COPY file, then input processing resumes with the original source

input file.

3.5.2 END

Syntax: [<label>] END

3.5.2.1 Fields

The label is optional and is  given  the  value  of  the  current

location counter if present.

3.5.2.2 Description

The END directive indicates to the assembler the end of the input

source  file.  This directive should be the last source statement

of the primary assembly file and should never be used in  a  COPY

file.

Texas Instruments            3-12

TI-74 ASM7                                    Further Information

SECTION 4

Further Information

For complete information on the TMS7000 assembly language, please

refer to the TMS7000 Assembly Language Programmer's Guide. (TI PN

1602127-9701)

For  information regarding the TMS7000 Series microprocessor

family,  refer  to  the  TMS7000  Family  Data  Manual.  (TI   PN

1603481-9701 rev B)

Either of these documents should be available from the local

TI Sales Office.

Texas Instruments            4-1

#